CSE421 Final Network Project Documentation

Multi-Zone Network Design using VLSM, Hybrid Routing, DHCP, DNS, HTTP, SMTP and POP3

Final Packet Tracer Topology Snapshot

Base Network: 10.109.0.0/16 | Student ID Method: Last 3 Digits = 109

This document consolidates the full project design, final addressing plan, topology, router links, DHCP pools, DNS/web/email services, routing configuration, verification steps, troubleshooting notes, member contributions and viva-ready explanations.

# Table of Contents

1. Project Overview and Requirements

2. Topology Overview

3. Base Network Derivation

4. VLSM Subnet Planning and Addressing

5. Device Inventory and Service Placement

6. DHCP Design and Pools

7. DNS, Web and Email Services

8. Routing Design and Router Commands

9. Verification and Testing Plan

10. Troubleshooting Notes

11. Viva Preparation Notes

12. Member Contribution Summary

13. Final Submission Checklist

# 1. Project Overview and Requirements

The project is a Cisco Packet Tracer implementation of a structured communication network connecting Malcolm’s School Lab, Francis’s Military Academy, Dewey’s Creative Experiment Lab, Reese’s Kitchen Control Zone, Hal’s Workplace Office and the Family Home network. The goal is full inter-zone communication with working services and clear verification.

| Requirement | Implemented Design |
| --- | --- |
| Base network from Student ID | Student ID 23101109 -> last three digits 109 -> 10.109.0.0/16 |
| VLSM subnetting | Subnets allocated from largest to smallest host requirement to reduce waste |
| Family Home services | DNSSrv-Home and WilkersonHub web server |
| Family Home addressing | Static IP addressing |
| DHCP zones | Router DHCP: Malcolm, Reese. Server DHCP: Francis, Hal, Dewey. Home: static. |
| Email service | EmailSrv-Malcolm and EmailSrv-Dewey using SMTP and POP3 |
| Direct router-level links | Malcolm-Hal and Francis-Reese direct links included; additional backbone links connect the full topology |
| Dynamic routing | RIP v2 used in the dynamic routing section |
| Static routing | Standard, default and floating static routes used for controlled paths and backup behavior |
| Verification | Ping, traceroute, DNS lookup, web browsing and email send/receive tests |

# 2. Topology Overview

The topology has six subnetted zones. Each zone has its own switch and router. End devices and servers connect to the local switch, and routers interconnect the zones through point-to-point /30 links. This design separates local LAN communication from inter-zone routing.

| Zone | Purpose | LAN Subnet | Gateway | Primary Devices |
| --- | --- | --- | --- | --- |
| Malcolm | School Lab | 10.109.0.0/22 | 10.109.0.1 | R-Malcolm, SW-Malcolm, PC-Malcolm1/2, EmailSrv-Malcolm |
| Francis | Military Academy | 10.109.4.0/23 | 10.109.4.1 | R-Francis, SW-Francis, PC-Francis1/2, DHCPSrv-Francis |
| Dewey | Creative Lab | 10.109.6.0/23 | 10.109.6.1 | R-Dewey, SW-Dewey, PC-Dewey1/2, EmailSrv-Dewey, DHCPSrv-Dewey |
| Reese | Kitchen Zone | 10.109.8.0/24 | 10.109.8.1 | R-Reese, SW-Reese, PC-Reese1/2 |
| Hal | Workplace Office | 10.109.9.0/25 | 10.109.9.1 | R-Hal, SW-Hal, PC-Hal1/2, DHCPSrv-Hal |
| Home | Family Home | 10.109.9.128/27 | 10.109.9.129 | R-Home, SW-Home, PC-Home1/2, DNSSrv-Home, WilkersonHub |

## 2.1 Router-to-Router Cable Choice

The router-to-router links are Ethernet links. Conceptually, a copper cross-over cable is used because router-to-router is a same-device-type Ethernet connection. A cross-over swaps transmit and receive pairs so TX connects to RX. Serial DTE was not selected because this project is LAN/Ethernet-based using GigabitEthernet ports, not a WAN serial design that requires DCE clock-rate configuration.

# 3. Base Network Derivation

| Item | Details |
| --- | --- |
| Student ID | 23101109 |
| Extraction method | Last 3 digits = 109 |
| Base network | 10.109.0.0/16 |
| Reason for /16 | A /16 gives 2^16 = 65,536 total addresses, enough for all LANs and point-to-point WAN links |
| Why not /24 | A /24 gives only 254 usable hosts, which cannot support Malcolm’s 840-host requirement |

# 4. VLSM Subnet Planning and Addressing

VLSM was calculated from largest to smallest host requirement. For each subnet, host bits were selected using 2^n - 2 >= required hosts. The CIDR prefix is then 32 - host bits.

| Network | Hosts Needed | Hosts + 2 | Block Size | Prefix | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Malcolm School Lab | 840 | 842 | 1024 | /22 | 1022 |
| Francis Military Academy | 390 | 392 | 512 | /23 | 510 |
| Dewey Creative Lab | 300 | 302 | 512 | /23 | 510 |
| Reese Kitchen Zone | 210 | 212 | 256 | /24 | 254 |
| Hal Workplace Office | 72 | 74 | 128 | /25 | 126 |
| Family Home Network | 25 | 27 | 32 | /27 | 30 |
| Router-to-router links | 2 each | 4 | 4 | /30 | 2 each |

## 4.1 Master IP Addressing Table

| Zone | Network | Prefix | Mask | Usable Range | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- | --- |
| Malcolm | 10.109.0.0 | /22 | 255.255.252.0 | 10.109.0.1 - 10.109.3.254 | 10.109.3.255 | 10.109.0.1 |
| Francis | 10.109.4.0 | /23 | 255.255.254.0 | 10.109.4.1 - 10.109.5.254 | 10.109.5.255 | 10.109.4.1 |
| Dewey | 10.109.6.0 | /23 | 255.255.254.0 | 10.109.6.1 - 10.109.7.254 | 10.109.7.255 | 10.109.6.1 |
| Reese | 10.109.8.0 | /24 | 255.255.255.0 | 10.109.8.1 - 10.109.8.254 | 10.109.8.255 | 10.109.8.1 |
| Hal | 10.109.9.0 | /25 | 255.255.255.128 | 10.109.9.1 - 10.109.9.126 | 10.109.9.127 | 10.109.9.1 |
| Home | 10.109.9.128 | /27 | 255.255.255.224 | 10.109.9.129 - 10.109.9.158 | 10.109.9.159 | 10.109.9.129 |

## 4.2 Point-to-Point Router Links

| Link | Network | Mask | Side A | Side B | Broadcast | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Malcolm-Hal | 10.109.9.160/30 | 255.255.255.252 | R-Malcolm 10.109.9.161 | R-Hal 10.109.9.162 | 10.109.9.163 | Active |
| Francis-Reese | 10.109.9.164/30 | 255.255.255.252 | R-Francis 10.109.9.165 | R-Reese 10.109.9.166 | 10.109.9.167 | Active |
| Reese-Malcolm | 10.109.9.168/30 | 255.255.255.252 | R-Reese 10.109.9.169 | R-Malcolm 10.109.9.170 | 10.109.9.171 | Active |
| Home-Francis | 10.109.9.172/30 | 255.255.255.252 | R-Home 10.109.9.173 | R-Francis 10.109.9.174 | 10.109.9.175 | Active |
| Reserved / optional | 10.109.9.176/30 | 255.255.255.252 | 10.109.9.177 | 10.109.9.178 | 10.109.9.179 | Reserved / not active in final topology |
| Home-Dewey | 10.109.9.180/30 | 255.255.255.252 | R-Home 10.109.9.181 | R-Dewey 10.109.9.182 | 10.109.9.183 | Active |

## 4.3 VLSM Tree Summary

10.109.0.0/16
├── 10.109.0.0/22     Malcolm School Lab       Gateway 10.109.0.1
├── 10.109.4.0/23     Francis Military Academy Gateway 10.109.4.1
├── 10.109.6.0/23     Dewey Creative Lab       Gateway 10.109.6.1
├── 10.109.8.0/24     Reese Kitchen Zone       Gateway 10.109.8.1
├── 10.109.9.0/25     Hal Workplace Office     Gateway 10.109.9.1
├── 10.109.9.128/27   Family Home Network      Gateway 10.109.9.129
├── 10.109.9.160/30   Malcolm ↔ Hal
├── 10.109.9.164/30   Francis ↔ Reese
├── 10.109.9.168/30   Reese ↔ Malcolm
├── 10.109.9.172/30   Home ↔ Francis
├── 10.109.9.176/30   Reserved optional link
└── 10.109.9.180/30   Home ↔ Dewey

# 5. Device Inventory and Service Placement

| Device Type | Devices Used | Purpose |
| --- | --- | --- |
| Routers | Cisco 2911 routers: R-Malcolm, R-Hal, R-Reese, R-Francis, R-Home, R-Dewey | Interconnect subnets, route traffic, provide router DHCP in selected zones |
| Switches | SW-Malcolm, SW-Hal, SW-Reese, SW-Francis, SW-Home, SW-Dewey | Connect end devices and servers inside each LAN |
| Servers | EmailSrv-Malcolm, EmailSrv-Dewey, DHCPSrv-Francis, DHCPSrv-Hal, DHCPSrv-Dewey, DNSSrv-Home, WilkersonHub | Application and network services |
| End devices | At least two PCs per zone | Connectivity, DHCP, email and web testing |

# 6. DHCP Design and Pools

The project uses mixed DHCP design. Malcolm and Reese use router-based DHCP pools. Francis, Hal and Dewey use dedicated DHCP servers. Home is static because it hosts stable infrastructure such as DNS and web services.

| Zone | DHCP Type | Pool / Range | Gateway | DNS |
| --- | --- | --- | --- | --- |
| Malcolm | Router-based DHCP | 10.109.0.11 onward | 10.109.0.1 | 10.109.9.130 |
| Reese | Router-based DHCP | 10.109.8.11 onward | 10.109.8.1 | 10.109.9.130 |
| Francis | Server-based DHCP | 10.109.4.11 - 10.109.5.254 | 10.109.4.1 | 10.109.9.130 |
| Hal | Server-based DHCP | 10.109.9.11 - 10.109.9.126 | 10.109.9.1 | 10.109.9.130 |
| Dewey | Server-based DHCP | 10.109.6.11 - 10.109.7.254 | 10.109.6.1 | 10.109.9.130 |
| Home | Static | Manual static addressing | 10.109.9.129 | 10.109.9.130 |

## 6.1 Router DHCP CLI: Malcolm

conf t
ip dhcp excluded-address 10.109.0.1 10.109.0.10
no ip dhcp pool MALCOLM-SCHOOL
ip dhcp pool MALCOLM
 network 10.109.0.0 255.255.252.0
 default-router 10.109.0.1
 dns-server 10.109.9.130
end
write memory

## 6.2 Router DHCP CLI: Reese

conf t
ip dhcp excluded-address 10.109.8.1 10.109.8.10
no ip dhcp pool REESE-KITCHEN
ip dhcp pool REESE
 network 10.109.8.0 255.255.255.0
 default-router 10.109.8.1
 dns-server 10.109.9.130
end
write memory

## 6.3 Server-Based DHCP Notes

Set DHCP service ON on DHCPSrv-Francis, DHCPSrv-Hal and DHCPSrv-Dewey.

Use the correct default gateway for the zone router interface.

Use DNS server 10.109.9.130 in every DHCP pool. Earlier 10.109.9.129 was wrong because that is the Home router, not the DNS server.

After changing a DHCP pool, renew each PC lease: Static -> DHCP or ipconfig /release and ipconfig /renew.

## 6.4 DHCP Relay Explanation

DHCP relay is required only when a DHCP client is in one subnet and the DHCP server is in another subnet. DHCP Discover messages are broadcasts and routers do not forward broadcasts. The relay router converts/forwards the request to the DHCP server using ip helper-address. In this project most DHCP servers serve their local subnet, so relay is not required for normal operation, but the command is important for viva.

interface g0/0
 ip helper-address <DHCP-server-IP>

# 7. DNS, Web and Email Services

## 7.1 DNS Server Configuration

| Item | Details |
| --- | --- |
| DNS server device | DNSSrv-Home |
| IP address | 10.109.9.130 |
| Subnet mask | 255.255.255.224 |
| Default gateway | 10.109.9.129 |
| DNS service | ON |

| DNS Record | Type | IP Address | Purpose |
| --- | --- | --- | --- |
| malcolm.com | A | 10.109.0.2 | Malcolm mail domain |
| mail.malcolm.com | A | 10.109.0.2 | Malcolm mail server hostname |
| dewey.com | A | 10.109.6.3 | Dewey mail domain |
| mail.dewey.com | A | 10.109.6.3 | Dewey mail server hostname |
| wilkersonhub.home.local | A | 10.109.9.131 | Web server hostname |

## 7.2 Web Server: WilkersonHub

| Item | Details |
| --- | --- |
| Server name | WilkersonHub |
| IP address | 10.109.9.131 |
| Subnet mask | 255.255.255.224 |
| Default gateway | 10.109.9.129 |
| DNS server | 10.109.9.130 |
| HTTP service | ON |
| Displayed page text | Life is unfair... but the network works! |

## 7.3 Email Server Configuration

| Server | IP | Gateway | DNS | Domain | Services | Accounts |
| --- | --- | --- | --- | --- | --- | --- |
| EmailSrv-Malcolm | 10.109.0.2/22 | 10.109.0.1 | 10.109.9.130 | malcolm.com | SMTP ON, POP3 ON | malcolm/1234, hal/1234, francis/1234, reese/1234 |
| EmailSrv-Dewey | 10.109.6.3/23 | 10.109.6.1 | 10.109.9.130 | dewey.com | SMTP ON, POP3 ON | dewey/1234, home/1234, wilkerson/1234 |

| Client Group | Email Address | Incoming/Outgoing Mail Server | Username / Password |
| --- | --- | --- | --- |
| Malcolm PCs | malcolm@malcolm.com | mail.malcolm.com | malcolm / 1234 |
| Hal PCs | hal@malcolm.com | mail.malcolm.com | hal / 1234 |
| Francis PCs | francis@malcolm.com | mail.malcolm.com | francis / 1234 |
| Reese PCs | reese@malcolm.com | mail.malcolm.com | reese / 1234 |
| Dewey PCs | dewey@dewey.com | mail.dewey.com | dewey / 1234 |
| Home PCs | home@dewey.com | mail.dewey.com | home / 1234 |

Email flow: sender PC uses SMTP to send to its mail server; the sender server delivers to the receiver domain server; receiver PC uses POP3 to fetch mail from its server. If DNS fails, email delivery fails even if ping by IP works.

# 8. Routing Design and Router Commands

The final routing uses a hybrid design. RIP v2 dynamically advertises learned routes between routing-domain routers. Static routes are used to force controlled paths to Home and Dewey. Default routes handle unknown destinations. Floating static routes provide backup paths with higher AD.

| Route Type | Purpose | Command Pattern | Project Example |
| --- | --- | --- | --- |
| Standard static | Manual route to a known destination network | ip route <net> <mask> <next-hop> | ip route 10.109.6.0 255.255.254.0 10.109.9.165 |
| Recursive static | Static route using next-hop IP; router resolves exit interface | same as standard static | Next-hop 10.109.9.165 is resolved through routing table |
| Default route | Forward all unknown traffic | ip route 0.0.0.0 0.0.0.0 <next-hop> | Home default route via 10.109.9.174 |
| Floating static | Backup path with higher administrative distance | ip route <net> <mask> <next-hop> <AD> | AD 25 backup route |
| Dynamic RIP v2 | Automatic route learning | router rip / version 2 / network 10.0.0.0 / no auto-summary | Used in dynamic routing section |

## R-Malcolm Router Configuration

conf t
hostname R-Malcolm
interface g0/0
 ip address 10.109.0.1 255.255.252.0
 no shutdown
interface g0/1
 ip address 10.109.9.161 255.255.255.252
 no shutdown
interface g0/2
 ip address 10.109.9.170 255.255.255.252
 no shutdown
router rip
 version 2
 network 10.0.0.0
 no auto-summary
exit
ip classless
ip route 10.109.9.128 255.255.255.224 10.109.9.162
end
write memory

## R-Hal Router Configuration

conf t
hostname R-Hal
interface g0/0
 ip address 10.109.9.1 255.255.255.128
 no shutdown
interface g0/1
 ip address 10.109.9.162 255.255.255.252
 no shutdown
interface g0/2
 no ip address
 shutdown
router rip
 version 2
 network 10.0.0.0
 no auto-summary
exit
ip classless
end
write memory

## R-Reese Router Configuration

conf t
hostname R-Reese
interface g0/0
 ip address 10.109.8.1 255.255.255.0
 no shutdown
interface g0/1
 ip address 10.109.9.166 255.255.255.252
 no shutdown
interface g0/2
 ip address 10.109.9.169 255.255.255.252
 no shutdown
router rip
 version 2
 network 10.0.0.0
 no auto-summary
exit
ip classless
ip route 10.109.6.0 255.255.254.0 10.109.9.165
end
write memory

## R-Francis Router Configuration

conf t
hostname R-Francis
interface g0/0
 ip address 10.109.4.1 255.255.254.0
 no shutdown
interface g0/1
 ip address 10.109.9.165 255.255.255.252
 no shutdown
interface g0/2
 ip address 10.109.9.174 255.255.255.252
 no shutdown
router rip
 version 2
 network 10.0.0.0
 no auto-summary
exit
ip classless
ip route 10.109.9.128 255.255.255.224 10.109.9.173
ip route 10.109.6.0 255.255.254.0 10.109.9.173
end
write memory

## R-Home Router Configuration

conf t
hostname R-Home
interface g0/0
 ip address 10.109.9.129 255.255.255.224
 no shutdown
interface g0/1
 ip address 10.109.9.173 255.255.255.252
 no shutdown
interface g0/2
 ip address 10.109.9.181 255.255.255.252
 no shutdown
ip classless
ip route 10.109.4.0 255.255.254.0 10.109.9.174
ip route 0.0.0.0 0.0.0.0 10.109.9.174
ip route 10.109.6.0 255.255.254.0 10.109.9.182 25
ip route 10.109.8.0 255.255.255.0 10.109.9.174
ip route 10.109.0.0 255.255.252.0 10.109.9.174
end
write memory

## R-Dewey Router Configuration

conf t
hostname R-Dewey
interface g0/0
 ip address 10.109.6.1 255.255.254.0
 no shutdown
interface g0/1
 ip address 10.109.9.182 255.255.255.252
 no shutdown
router rip
 version 2
 network 10.0.0.0
 no auto-summary
exit
ip classless
ip route 0.0.0.0 0.0.0.0 10.109.9.181
ip route 10.109.9.128 255.255.255.224 10.109.9.181
ip route 10.109.8.0 255.255.255.0 10.109.9.181
ip route 10.109.0.0 255.255.252.0 10.109.9.181
end
write memory

# 9. Verification and Testing Plan

| Test Area | Command / Action | Expected Result |
| --- | --- | --- |
| Interface status | show ip interface brief | All active router interfaces show up/up. Unused interfaces may be shutdown. |
| Routing table | show ip route | Connected, RIP and static routes appear correctly. |
| RIP status | show ip protocols | RIP v2, no auto-summary, routing network 10.0.0.0. |
| DHCP leases | show ip dhcp binding / PC ipconfig | PCs receive correct IP, gateway and DNS. |
| DNS | nslookup mail.dewey.com / ping dewey.com | Domain resolves to 10.109.6.3. |
| Web | Browser -> http://wilkersonhub.home.local | WilkersonHub page loads. |
| Email | Send Malcolm/Reese -> dewey@dewey.com then Receive on Dewey PC | Email appears in receiver inbox. |
| Path check | traceroute 10.109.6.3 | Shows routing path through backbone. |

## 9.1 Core Commands

PC Command Prompt:
ipconfig /all
ping 10.109.0.1
ping 10.109.6.3
ping dewey.com
tracert 10.109.6.3

Router CLI:
show ip interface brief
show ip route
show ip route rip
show ip route static
show ip protocols
show running-config | include ^ip route
show running-config | section router rip

# 10. Troubleshooting Notes

| Problem Observed | Meaning | Fix |
| --- | --- | --- |
| ping IP works but ping domain fails | Routing works, DNS resolution broken | Add DNS A records and ensure PCs use DNS 10.109.9.130 |
| Destination host unreachable | A router does not know a route to destination or next-hop is invalid | Check show ip route and correct static/RIP route |
| Request timed out | Packet sent but no reply returned | Check destination IP, gateway, service, cable and return route |
| First ping .!!!! then next ping !!!!! | ARP resolution caused first packet loss | Normal behavior; retest |
| DHCP gives DNS 10.109.9.129 | Wrong DNS in DHCP pool. 10.109.9.129 is router, not DNS server | Change DHCP DNS to 10.109.9.130 and renew lease |
| Multiple DHCP pools on same network | Clients may receive wrong settings | Delete duplicate pool and keep only correct pool |
| Packet Tracer Bluetooth overlap on PC | IP assigned to Bluetooth instead of FastEthernet | Disable/remove Bluetooth or recreate PC; use FastEthernet only |
| Email delivery failure: domain not found | Mail server cannot resolve destination domain | Add dewey.com/mail.dewey.com and malcolm.com/mail.malcolm.com DNS records |

# 11. Viva Preparation Notes

### Why use VLSM?

Each zone has different host requirements. VLSM lets us allocate different subnet sizes and reduce address waste.

### What does /22 mean?

/22 means 22 network bits and 10 host bits. Total addresses = 2^10 = 1024; usable hosts = 1022.

### Why RIP v2?

RIP v2 supports classless routing and VLSM. It automatically exchanges routes using hop count.

### Why no auto-summary?

Auto-summary can break classless subnetting. no auto-summary keeps the actual subnet masks.

### When use static routing?

Use static routing for controlled, predictable paths and small/stable network sections.

### What is a default route?

A route for unknown destinations: 0.0.0.0/0. It forwards packets when no more specific route exists.

### What is floating static route?

A backup static route with higher AD. It is used only when the primary route is unavailable.

### What is default gateway?

The router IP a host uses to reach destinations outside its own subnet.

### What is passive-interface?

It stops RIP updates from being sent out an interface, usually toward end-device LANs.

### How does DHCP work?

DORA: Discover, Offer, Request, Acknowledge. It provides IP, mask, gateway and DNS.

### What is DHCP relay?

A router forwards DHCP broadcasts to a DHCP server in another subnet using ip helper-address.

### What does DNS do?

DNS translates names such as mail.dewey.com into IP addresses such as 10.109.6.3.

### What protocols are used for email?

SMTP sends email; POP3 retrieves email from the receiver mail server.

### What protocol is used for web browsing?

HTTP is used for browser-to-web-server communication.

### What does ping test?

Ping uses ICMP to check reachability between devices.

### What does traceroute show?

Traceroute shows the path and hops packets take to reach a destination.

### Why cross-over cable between routers?

Router-to-router Ethernet is same-device type; cross-over swaps transmit and receive pairs.

### Why not Serial DTE?

Serial DTE is for WAN serial links requiring DCE clocking. This project uses GigabitEthernet LAN links.

# 12. Member Contribution Summary

| Member / Section | Contribution |
| --- | --- |
| Maisha | Base network derivation, host requirement ordering, VLSM calculation, subnet assignment, VLSM tree and master addressing table. |
| Shafin | Router model selection, topology routing structure, WAN router interface planning, RIP/static routing CLI and path analysis. |
| Fardin Saurov | Network summary, PC addressing, DHCP type mapping, DHCP pool fixes, DNS/gateway validation and client-side DHCP renewal instructions. |
| Jahid | DNS records, web/mail domain design, EmailSrv-Malcolm and EmailSrv-Dewey configuration, PC email client setup and email testing plan. |

# 13. Final Submission Checklist

Topology screenshot has clear labels for routers, switches, servers and PCs.

VLSM tree is included and matches the master IP table.

All LAN and router-to-router subnets are documented.

Router CLI commands are included for every router.

Router-based DHCP and server-based DHCP zones are clearly identified.

DNS records for malcolm.com, mail.malcolm.com, dewey.com, mail.dewey.com and wilkersonhub.home.local are configured.

Web server WilkersonHub displays: Life is unfair... but the network works!

Email send/receive works using SMTP and POP3.

Ping and traceroute tests prove full connectivity.

Known Packet Tracer interface conflicts have been cleaned or documented.